STS and XTS Telephone System Quick Reference Sheet				by DDK Telecommunications

FIXED FEATURE BUTTONS

FLASH -- lets you terminate an outside call and restore dial tone without having to hang up the handset. It is also used to transfer calls behind a PBX or Centrex system.

TRANS (TRANSFER) -- lets you transfer an outside call from one station to another.

SPEED -- provides access to speed dialing, save number redial and last number redial. It’s also used for flexible button programming.

CONF (CONFERENCE) -- used to establish conference calls. See detailed information below.

MSG (MESSAGE WAIT) -- allows you to initiate a message waiting indication at a station that is unattended or in Do Not Disturb.

FWD (FORWARD) -- allows you to forward your calls to another station or location. The Forward button also doubles as the DND button.

MUTE -- allows you to switch the built-in microphone on or off when using the speakerphone. Also controls the handset microphone when using the handset.

CAMP ON -- enables you to alert a busy party that an outside line is on hold for them. Call the busy station and press the Camp On button. A beep tone will be heard at the recipient’s phone. The recipient can press their CAMP ON button to be connected to the calling party.

ON/OFF (SPEAKER PHONE) -- turns the telephone on and off when using the speakerphone or the headset.

HOLD (SAVE) -- enables you to place an outside caller on hold. The Hold button also doubles as the SAVE or ENTER button. See below.

VOLUME BAR -- allows you to adjust the volume for your speakerphone, handset, and ringing volume. Also adjusts the contrast of the display.

H-T-P -- allows you to select the intercom signaling mode: Hands-free, Tone ring, or Privacy hands-free.

DESCRIPTIONS & EXPLANATIONS

PLACING AN OUTSIDE CALL

To make an outside call press the POOL button, press 9 or press a direct line appearance button.

PLACING AN OUTSIDE LINE ON HOLD

While connected to an outside line, press the HOLD button once for system-wide hold or double click on the hold button to place a call on exclusive hold which only allows you to retrieve the call. System-wide hold allows anyone on the phone system to pick up that call. The line button that’s holding will flash green at your phone and red at everyone else’s phone.

UNSCREENED TRANSFER

While connected to an outside line:

1. Press TRANS button and dial station number or press the station’s DSS (Direct Station Select) button on your phone.

2. When the called extension is signaled, hang up to transfer the call or press the ON / OFF button.

SCREENED TRANSFER

While connected to an outside line:

1. Press TRANS button and dial station number or press the station’s DSS (Direct Station Select) button on your phone.

2. When the person at the called extension answers, announce the caller (i.e. You have a call from John Adams).

3. Hang up to complete the transfer by replacing the handset or pressing the ON / OFF button.

ESTABLISHING A CONFERENCE

1. Lift the handset or press the ON / OFF speakerphone button.

2. Call another station or pick an outside line and call your desired party. When called party answers, tell them to hold on and press CONF.

3. Add the next conference party by selecting another outside line or intercom station.

4. Press CONF button again and repeat previous step in a cycle, until all parties are contacted.

5. When the last party answers, press the CONF button twice. All parties will be connected.

TERMINATING A CONFERENCE

To end a Supervised Conference, the conference initiator replaces the handset or presses ON / OFF.

To end an Unsupervised Conference, press the flashing CONF button while on-hook.

CALL FORWARDING

1. Lift the handset or press the On / Off speaker phone button.

2. Press the FWD button.

3. Dial the desired Call Forward code:

[6] = All Calls via Unconditional    [7] = No Answer    [8] = Busy    [9] = Busy/No Answer

followed by the station number or the destination where calls are to be forwarded. Confirmation tone sounds.

4. Replace the handset or press ON / OFF button.

To Cancel Call Forward press ON / OFF button FWD button ON / OFF button.

LEAVING A MSG WAITING INDICATION

If you dial a station that is unattended or in Do Not Disturb, you can leave a message waiting indicator light.

1. Press the ON / OFF button.

2. Dial the desired intercom station.

3. Press the MSG button. The called party’s MSG button LED will flash slowly.

4. Press the ON / Off button to end the call.

INITIATING A CALL BACK REQUEST

Call Back is used when you call a busy extension.

1. Lift the handset or press the ON / OFF speakerphone button.

2. Call another station.

3. When you hear the busy tone, press the Call Back button and hang up.

4. When the other person finishes his or her call and hangs up the phone, you will be Called Back and notified that they hung up.

5. Simply lift your handset and that person will be immediately contacted via the intercom.

Note: On some phones the Call Back feature may be accessed by pressing the right soft touch button next to the display.

INITIATING A CAMP ON AND GENERATING A CALL WAITING TONE

Camp On is used when you call a busy extension.

1. Lift the handset or press the ON / OFF speakerphone button.

2. Call another station.

3. When you hear the busy tone, press the CAMP ON button. The called party will hear a call waiting beep tone, “Camp-on from ext XXX” will be displayed on their phone and their Camp On light will be flashing.

4. The called party may press his or her CAMP ON button. The current caller will be placed on hold and they will be connected to the Camped-On party via intercom.

5. The called party may now press the flashing line on hold to return to the original caller.

Note: On some phones the Camp On feature may be accessed by pressing the right soft touch button next to the display.

OFF HOOK VOICE OVER (OHVO): If you call a busy telephone station, this button allows you to speak in the person’s ear even though they are on the phone with another party. The other party will not hear your voice. Only the person you’re calling will hear your voice. The person you’re calling can then respond to you by hitting their mute button. The mute button will toggle between the two callers.

STORING SPEED NUMBERS

1. Press the SPEED button twice.

2. Dial the speed bin location.

    - 9000 to 9019 = Station speed numbers

    - 9020 to 9099 = System wide speed numbers (performed at attendant’s phone)

3. Dial the desired telephone number and press the HOLD button to save.

4. Press ON/OFF (Speaker) to end programming.

DIALING A SPEED NUMBER

1. Press SPEED and dial the speed bin location or press the preprogrammed speed bin button.

    - 9000 to 9019 = Station speed numbers

    - 9020 to 9099 = System wide speed numbers

2. When the called party answers, pick up the handset or use the speakerphone.

REDIAL:  Press SPEED then #.

DIRECTORY DIALING

Users with display telephones may view a list of individuals on the telephone system and have the system automatically dial that person. Directory Dialing may also be used to transfer a call from one station to another.

1. Press directory list flex button or enter code 680.

2. Press a button on the keypad once, twice, or three times, that represents the letter of the alphabet, to begin viewing the list of names.

3. Using the * and # buttons, scroll through the Directory List to select a name.

4. When the selected name is highlighted, press the SPEED button to automatically dial the destination station or outside phone number.

PROGRAMMING FLEXIBLE BUTTONS

1. Press the SPEED button twice.

2. Press the flexible button to be programmed.

3. Dial the desired feature code or station number from the chart found on page 100 of your user guide. Confirmation tone sounds.

4. Press the ON / OFF button.

To erase a flexible button:

1. Press the SPEED button twice.

2. Press the flexible button to be programmed.

3. Press the FLASH button. Confirmation tone sounds.

4. Press the ON / OFF button.

PAGING

1. Lift the handset.

2. Enter the page number from the following list:

    - 700 -- Pages all zones both internally to phones and externally to paging speakers

    - 721 -- Pages all phones

    - 701 - 720 -- Internally pages a specific group of phones. (Zones 1 - 20)

    - 761 -- External page to ceiling paging speakers

3. Make your announcement. Hang up the handset.

PARKING A CALL

Parking a call is similar to placing a call on hold.

While connected to an outside line:

1. Press the TRANS button.

2. Enter the 3-digit Call Park location (424 - 437) and then hang up.

3. Now make a page or intercom the recipient and inform them that they have a call parked at 424 through 437.

4. To retrieve the Parked Call, go to any phone and press 424 through 437.

For more information, go to www.ddktele.com

VOICE MAIL: Press Voice Mail (VM) to access your own voice mailbox. One can also use the Voice Mail button to send an outside caller to someone’s Voice Mailbox. After answering a call, simply press Voice Mail, enter the 3-digit extension for the mailbox you wish to reach and hang up.

LOOP: Incoming calls will be received on either a Direct Line Appearance button, (i.e. Line 01, Line 02, Line 20, etc.) or if a call comes in on a C.O. line (Central Office Line) which does not appear on your phone, it will ring in on one of your LOOP buttons.

HEADSET / HANDSFREE: This button will toggle your ON / OFF button from being used for a hands free speaker phone call to being used for a call to your telephone headset. This may be programmed on any button on your phone but is frequently programmed as your own extension number.

PICKUP is used to intercept a call ringing at any station within your pickup group. (i.e. An intercom call is ringing to another desk but that person is at lunch. Simply press your Pickup button or press # then 0 and the call is diverted to your phone).

DND (Do Not Disturb) -- places your phone in Do Not Disturb mode and blocks incoming calls or on an Attendant phone will change the ring pattern of incoming calls.

CID Name (Caller ID Name) is used to set your phone display to show the name as well as the number of the incoming caller. It may be turned on or off at any time. Caller ID name and number is an option which needs to be turned on by the local telephone line provider.

Vodavi Feature Codes that you should know in case your phone acts up

By DDK Telecommunications

Vodavi uses three digit codes to activate or deactivate various features on their phone systems. Since Vodavi’s 3-digit codes may be the same as your local calling exchange or prefix, this occasionally becomes a problem. This is an example of what may occur: Let’s say the cleaning person walks up to a phone and decides to call her son at home. He lives in Williamsville at 631-xxxx. Therefore, she picks up the handset, hears an intercom dial tone, dials 631-xxxx, and hears a beep in her ear. She just inadvertently activated Do Not Disturb on the phone since she didn’t know enough to first choose an outside line and listen for a dial tone. Here’s another example: Let’s suppose a salesperson asks to use your office phone. He picks up the phone, hears an intercom dial tone instead of an outside line dial tone and calls 644-yyyy. Your phone now displays “STA IN: OUT OF OFFICE”. Therefore, as a Vodavi phone user you need to know how to clear these feature codes. Do not lift the handset or press ON/OFF when entering the code.

662	this code clears call forwarding activated by codes 640 & 642, personal text messages on your display activated by code 633yy and Do Not Disturb.

644	this code clears the message “STA IN: OUT OF OFFICE”. If this code is activated, you will receive intercom calls but not receive any calls through your auto-attendant voice mail system.

631	this code activates or deactivates “Do Not Disturb”. Do not disturb will either block all calls to your phone or may simply block paging to your phone.

634	this code activates or deactivates Headset mode at your phone. In headset mode your phone’s On/Off button will no longer turn the speakerphone On and Off, it will turn a headset On and Off.

632	this code activates or deactivates Background Music at your phone. This is generally the same audio source as the Music On Hold.

695	this code changes the ringing sound of your phone. You change the ring tone by pressing 695xx where xx is any number from 00 to 36. To change your phone back to the default ring, press 69500.

607	this code can be used by an attendant phone to set the phone in an unavailable status. To place the phone in an available status, simply dial 607.

691	this code sets an off-hook preference button on your phone. The flexibly programmable buttons on your phone are numbered from top left to bottom right. The button at the top left is button 01. Generally the button to the right is button 02. Example: Let’s say you wanted your phone to automatically select line 4 every time you lifted the handset. You would count down your phone to line 4. If it were button number 16, you would simply press 69116 into your touch-tone pad and every time you lifted your handset, it would grab line 3. The disadvantage to this is you will no longer hear an intercom dial tone when you lift your handset. Therefore, if you need to intercom someone, you will need to enter his or her extension BEFORE lifting your handset. To clear the off-hook preference button, dial 69100 without lifting the handset.

636	this is the code for station relocation and is probably the most dangerous code to play with. Most systems will have this code disabled by the system administrator but it is active by default from Vodavi. This feature, if used, would allow a station user to take their phone to another extension’s jack in the building, dial 636, then their old 3-digit extension number, and have the system copy the features and extension number to the new jack port location. The use of this feature should be avoided. It can cause erratic behavior in the phone system. It will drive your phone vendor insane because when you report a problem with extension 139 (which has been moved to jack (port) 182),  he will have no clue that you did this and will spend hours tracing and working on jack (port) 139 because by default this is supposed to be extension 139. Worst of all, if your bosses phone extension 108 was moved to jack (port) 139, he will be turning your bosses phone on and off 15 times trying to figure out why extension 139 is still on. Utilizing this feature without clear documentation posted across the front of the phone system is a good way to get fired as well as receive a hefty bill from your phone vendor.